# Fit Session Report

Ellory & Vane  ·  Bespoke & Made-to-Measure  ·  No. 14 Savile Row, London

| Garment ref. | EV-27J-0142 / Single-breasted 2-button lounge jacket |
| --- | --- |
| Style no. | AW27-BR-JKT-014 (“The Marlborough”) |
| Cloth | Fox Brothers 13oz charcoal herringbone, 100% worsted wool (lot 8842) |
| Sample size | 40R · block: EV House Draft B (drop 6) |
| Fitter | J. Ellory (Head Fitter) · Cutter: T. Vane |
| Model / fit spec | House fit form 40R + client last (D. Ashcombe) · standard posture |
| Fitting | First basted fitting · Session 1 of 3 |
| Date / season | 14 July 2026 · Autumn/Winter 2027 |

| OVERALL VERDICT REVISE | Sample is sound in cloth and construction but not yet passable: three points of measure sit outside working tolerance and the balance reads front-heavy. Corrections are contained and achievable inside one iteration — re-cut of the sleeve, a waist let-out, and a gorge/collar re-set — so the piece is returned for a second basted fitting rather than rejected. |
| --- | --- |

## Fit summary

On the form the jacket hangs clean through the chest and the herringbone matches at the centre-back seam and across both quarters. The revise call is driven by fit geometry, not make. The waist is a full 1.4 cm over spec on the half-measure, collapsing the intended suppression and letting the front blades stand away; the right sleeve reads 1.3 cm short from the centre-back, breaking the cuff line against the shirt; and the gorge has crept 0.9 cm high and open, pushing the lapel roll off the intended break. None of these is structural — each is a marking or pressing correction — but together they carry the sample below a passable first fitting. Detailed measures and zone commentary follow.

# Points of Measure — measured against spec

All values in centimetres on the half-measure unless noted. Deviation = sample − spec. A point is flagged OUT OF TOL when |deviation| exceeds the stated tolerance.

| POM | Description | Spec | Sample | Tol ± | Dev. | Status |
| --- | --- | --- | --- | --- | --- | --- |
| A01 | Chest, 1/2 (2.5 cm below armhole) | 52.0 | 52.4 | 0.6 | +0.4 | ● OK |
| A02 | Waist, 1/2 (at fore-button) | 47.0 | 48.4 | 0.8 | +1.4 | ● OUT OF TOL |
| A03 | Hem sweep, 1/2 | 54.0 | 54.3 | 0.8 | +0.3 | ● OK |
| B01 | Shoulder, seam to seam | 45.0 | 45.2 | 0.5 | +0.2 | ● OK |
| B02 | Across back, 1/2 | 20.5 | 20.6 | 0.5 | +0.1 | ● OK |
| C01 | Sleeve length, CB to cuff (R) | 84.5 | 83.2 | 0.6 | −1.3 | ● OUT OF TOL |
| C02 | Cuff opening | 14.5 | 14.6 | 0.4 | +0.1 | ● OK |
| C03 | Bicep, 1/2 | 20.0 | 20.3 | 0.5 | +0.3 | ● OK |
| D01 | Back length, HPS to hem | 76.0 | 76.4 | 0.6 | +0.4 | ● OK |
| D02 | Front length, HPS to hem | 74.5 | 74.7 | 0.6 | +0.2 | ● OK |
| E01 | Gorge height, neck-point to notch | 8.5 | 9.4 | 0.4 | +0.9 | ● OUT OF TOL |
| E02 | Lapel width at widest | 8.5 | 8.6 | 0.3 | +0.1 | ● OK |

## Corrections requested

A02 — Waist.  Let out is not the fix; the suppression was marked shy. Re-mark the side-body and back seams to recover 1.4 cm of shape at the fore-button and re-press the waist curve. Confirm the drape closes over the hip.

C01 — Sleeve length (R).  Right sleeve set 1.3 cm short. Re-cut from the sleeve-head reserve and re-hang; verify pitch at the same time (see zone notes). Left sleeve is in tolerance — do not disturb.

E01 — Gorge / collar.  Gorge is high and open by 0.9 cm. Drop the notch and re-set the collar to lower the break point one button; re-press the lapel roll to a soft 3-roll-2.

# Balance & Aesthetic — by body zone

### Collar & neckline

The collar sits close at the centre-back with no gap, but rides high on the right neck-point where the gorge has crept. Once the gorge is re-set the collar band should settle a clean 1.5 cm of shirt-collar showing. No wrinkling at the fold line.

### Shoulder & back balance

Shoulder line is clean and the pad sits without divot; across-back is in tolerance. A faint diagonal drag runs from the right shoulder-blade toward the under-arm — consistent with the short right sleeve pulling the scye — and should release once the sleeve is re-hung.

### Chest & lapel roll

Chest is full and honest with no break under the arm. The lapel roll is the weak point: the high gorge forces the break above the intended point, standing the lapel proud of the chest. Re-setting the gorge and pressing a soft roll recovers the line.

### Waist suppression

The dominant fault. At 1.4 cm over spec the waist is effectively un-suppressed, so the fronts stand away and the skirt loses its intended clean sweep over the seat. This is a marking correction, not a let-in, and drives the revise verdict.

### Sleeve pitch & length

Right sleeve is both short (−1.3 cm) and pitched a touch forward, throwing a soft fold on the fore-arm. Re-cut for length and re-hang to correct pitch in one operation. Left sleeve hangs true — use it as the reference.

### Skirt, vents & hem

Double vents hang closed at rest and the hem is level within tolerance. The front quarters open slightly at the hem, a downstream symptom of the waist rather than a hem fault; re-assess after the waist is taken in before touching the vents.

## Fit record — first basted fitting

|  | [ photo pending ] | [ photo pending ] |
| --- | --- | --- |
| FRONT — sample worn | SIDE — to follow | BACK — to follow |

Front image: client D. Ashcombe in the basted sample, House studio, 14 Jul 2026. Side and back frames to be added at the second fitting.

# Revisions for the next sample

Actions required before the second basted fitting. Numbering is Word-driven so the list renumbers automatically as items are added or removed.

Re-mark waist suppression on side-body and back seams to recover 1.4 cm of shape; re-press the waist curve and confirm the fronts close over the hip.

Re-cut the right sleeve to full length from the sleeve-head reserve and re-hang, correcting forward pitch in the same operation; leave the left sleeve untouched.

Drop the gorge and re-set the collar to lower the break one button; re-press the lapel to a soft 3-roll-2 and verify the roll sits on the chest.

Re-assess the front-quarter hem opening only after the waist is taken in; adjust the hem sweep to spec if the opening persists.

Re-press the right shoulder-blade drag and confirm it releases once the sleeve is re-hung; check the scye for residual strain.

Prepare side and back fit frames and a fresh half-measure sheet for all twelve POMs at the second fitting; carry this report for sign-off comparison.

## Ownership & due dates

| # | Action | Owner | Due |
| --- | --- | --- | --- |
| 1 | Waist re-mark & re-press | T. Vane (Cutter) | 18 Jul 2026 |
| 2 | Right sleeve re-cut & re-hang | T. Vane (Cutter) | 18 Jul 2026 |
| 3 | Gorge / collar re-set | J. Ellory (Fitter) | 19 Jul 2026 |
| 4 | Hem sweep re-assess | A. Reeve (Tailor) | 20 Jul 2026 |
| 5 | Shoulder drag re-press | A. Reeve (Tailor) | 20 Jul 2026 |
| 6 | Frames & measure sheet prep | Studio | 21 Jul 2026 |

## Sign-off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Fitter | J. Ellory | x ____________________ | _________ |
| Head Cutter | T. Vane | x ____________________ | _________ |
| Client approval | D. Ashcombe | x ____________________ | _________ |

Return the corrected sample with this report attached. Second basted fitting to be scheduled on completion of the actions above.